NETWORKING HOME LAB
PORTFOLIO

Hands-on configuration • Verification • Troubleshooting

| Project 1 | MikroTik Routing and VLAN Lab |
| --- | --- |
| Project 2 | Ubuntu Server and WireGuard VPN |
| Project 3 | UniFi Wireless Network Integration |
| Project 4 | Cisco Packet Tracer Networking Labs |

Prepared as a practical learning portfolio for entry-level Network Technician, IT Support, and Junior Network Engineer opportunities.

This document records practical lab work and the troubleshooting methods used. It distinguishes confirmed observations from incomplete work.

Project 3 — UniFi Wireless Network Integration

Wireless networking | Ubiquiti UniFi AP-AC-Pro and MikroTik hAP ac lite

1. Project Overview

Connected and tested a UniFi AC Pro access point on a MikroTik hAP ac lite network. Verified that the AP obtained a LAN address, responded to ping, and broadcast Wi-Fi. Also investigated UniFi management/adoption and inform communication issues.

2. Objectives

Connect a UniFi AP to the existing router LAN and confirm link/IP connectivity.

Access the AP for diagnostics using SSH and inspect its device information.

Check wireless broadcasting and management/controller status.

Understand that assigning VLANs to SSIDs requires matching AP settings and switch/router trunk configuration.

3. Network Topology

Internet → MikroTik hAP ac lite (wireless uplink/LAN) → Ethernet → UniFi UAP-AC-Pro. Windows workstation is used for ping, browser management, and SSH diagnostics. Additional SSID-to-VLAN mapping is a planned extension and must not be assumed operational.

4. Configuration and Commands

The following are examples/reference commands. Adapt interface names, IPs, VLAN IDs, and device versions to the configuration you actually tested.

Windows reachability checks

ping <AP_IP>
arp -a
Test-NetConnection <AP_IP> -Port 22

UniFi AP information command (from AP SSH)

info

Controller inform URL example from this lab

http://192.168.88.254:8080/inform
Use only if this is still the correct controller address and port in your current network.

5. Verification and Testing

Check the AP link and DHCP/IP assignment in the router or client list.

Ping the AP IP from the Windows workstation and verify the ARP entry.

Confirm the AP management page/status is reachable and Wi-Fi is broadcasting.

Check controller adoption state and inform URL; verify the controller is listening on the required ports.

6. Troubleshooting Experience

Verified that a UniFi AP at a 192.168.110.x address responded to ping and was reachable from the Windows workstation.

Investigated repeated inform/decryption messages and controller adoption visibility.

Investigated a Windows port conflict affecting UniFi Network Server's inform port; verified the service later bound the required ports.

7. Final Result / Current Status

AP LAN reachability and Wi-Fi broadcasting were verified during the lab. Controller adoption and VLAN-backed main/guest SSID segmentation .

8. What I Learned

Difference between AP IP reachability, controller adoption, and successful client traffic.

How to use ping, ARP, SSH, device information, and controller logs together.

SSID-to-VLAN segmentation needs consistent settings across the AP and upstream network.

9. Real-World Application

Relevant to small-office Wi-Fi deployment, AP onboarding, controller troubleshooting, and guest/main wireless separation.

10. Evidence Checklist

- Topology diagram with labels and IP/VLAN plan

- Configuration screenshot or sanitized command output

- Verification output showing successful tests

- Troubleshooting evidence: symptom → investigation → fix → retest

- Date, device/software version, and brief caption

- For more information about my troubleshooting experience , you may see it to my files thank you!